📘 SOFTWARE REQUIREMENTS SPECIFICATION (SRS)

1. Introduction

1.1 Purpose

The purpose of this project is to design and implement a Hybrid Edge–Cloud Intelligent Risk Detection System that can detect risky situations using a camera, process data locally on the edge (laptop), and trigger real-world actions using an ESP32-controlled relay.
The system focuses on real-time response, offline functionality, and practical hardware integration, making it suitable for real-world safety and automation scenarios.

1.2 Scope

The system captures live video from a camera, performs basic vision-based detection on the laptop, classifies the situation into predefined risk levels, and communicates the risk to an ESP32 microcontroller via serial communication.
Based on the received risk level, the ESP32 controls a relay module to switch external devices (alarm, light, cutoff switch).
An optional cloud (MQTT) component is included for monitoring and logging but is not mandatory for core operation.

1.3 Definitions & Abbreviations

| Term | Description |
| --- | --- |
| Edge Computing | Processing data locally without cloud dependency |
| ESP32 | Microcontroller with Wi-Fi & GPIO support |
| Relay | Electromechanical switch controlled electronically |
| MQTT | Lightweight publish–subscribe protocol |
| Risk Level | System-defined severity (LOW / HIGH) |

2. Overall Description

2.1 Product Perspective

The system is a standalone embedded + software solution that integrates:

- Camera-based detection (Laptop)

- Hardware control (ESP32 + Relay)

- Optional cloud connectivity (MQTT)

It is designed to work independently of internet availability, with the cloud used only for extended features.

2.2 User Classes

| User | Description |
| --- | --- |
| Operator | Starts and monitors the system |
| Administrator | Configures thresholds and hardware |
| Viewer | Observes system status (optional UI) |

2.3 Operating Environment

- Hardware: Laptop with camera, ESP32, Relay Module

- Software: Python, Arduino IDE

- OS: Windows / Linux

- Connectivity: USB Serial, Optional Wi-Fi

2.4 Design & Implementation Constraints

- Limited hackathon time

- Basic computer vision (non-AI by default)

- Serial communication bandwidth

- Hardware safety limitations

3. System Architecture

3.1 High-Level Workflow

- Camera captures live video frames

- Laptop processes frames at the edge

- Risk level is generated

- Risk is sent to ESP32 via serial

- ESP32 activates relay based on risk

- Optional: event published to cloud

3.2 Architecture Characteristics

- Real-time processing

- Offline-first design

- Modular components

- Hardware–software integration

4. Functional Requirements

FR1: Camera Input

The system shall capture real-time video from a connected camera.

FR2: Edge Processing

The system shall analyze video frames locally to detect abnormal activity.

FR3: Risk Classification

The system shall classify detected activity into predefined risk levels.

FR4: Serial Communication

The system shall transmit risk levels to the ESP32 using serial communication.

FR5: Relay Control

The system shall activate or deactivate the relay based on the received risk level.

FR6: Offline Operation

The system shall continue functioning without internet connectivity.

FR7: Cloud Monitoring (Optional)

The system may publish risk events to an MQTT broker.

5. Non-Functional Requirements

- Performance: Detection latency < 1 second

- Reliability: System must not crash during continuous operation

- Usability: Simple startup and clear status indication

- Scalability: Can be extended to multiple sensors

- Maintainability: Modular code structure

6. Hardware Requirements

- ESP32 Development Board

- Relay Module (1-channel)

- Laptop with Camera

- USB Cable

- External Load (buzzer/light)

7. Software Requirements

- Python 3.x

- OpenCV Library

- pySerial Library

- Arduino IDE

- ESP32 Board Package

8. Use Case (Brief)

Use Case: Automatic Hazard Response

- Actor: System

- Trigger: High-risk detection

- Action: Relay activation

- Outcome: Immediate physical response

9. Limitations

- Basic detection logic (not advanced AI)

- Single camera support

- Limited environmental testing

10. Future Enhancements

- AI-based detection models

- Multiple camera integration

- Web dashboard

- SMS / Mobile alerts

- Cloud analytics

11. Conclusion

This project demonstrates a practical, real-time safety system using edge computing and embedded hardware.
By combining camera-based detection with ESP32-controlled relay actions, the system delivers immediate response, offline reliability, and scalable architecture, making it suitable for real-world deployments and hackathon evaluation.